REPORT summary for LR tests on AWS

| Sawtooth version | Test Config | Platform | Consensus | Run time | Measured Block Rate | Measured Transaction Rate | Failed/ Restarted Nodes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - release branch (apt repo ubuntu/bumper/nightly) | LR7(AWS) | AWS | PoET Simulator | 168h | 85.83/h | 9.1 | None |

LR7(AWS): 10 validator nodes, 10 TPS applied transaction rate(intkey 5TPS + smallbank 5TPS)

EC2: m4.2xlarge ( 8 CPUs, 32GB RAM), 100 GB SSD 5000 IOPS

LR on AWS

Input parameters

Sawtooth: fetched from ubuntu/bumper/nightly apt repo (sawtooth-core debians =1.1.1~dev1201-1, sawtooth-poet debians=1.1.2~dev37-1)

I/P Workload: 5 TPS intkey + 5 TPS smallbank

Mode: Simulator

Observations

- Ran for 7 days

- Output observed as 9.1 TPS

- Total blocks

  - 15371 blocks = 168 hours

  - 91.49 blocks = 1 hour

  - 1 block = 49.34 seconds against 30 seconds/block

- Blocks published deviation is 164 i.e. difference of max number of blocks (2364) published by a node to min number of blocks (2200) published by a node.